# Autonomous Rocket Flight and Landing Simulation

Problem Statement:

The objective of this project is to simulate the vertical motion of a rocket and design a control system that allows the rocket to reach a desired altitude autonomously.

The system is modeled using basic physics and implemented in MATLAB using numerical simulation.

Assumptions:

To simplify the analysis, the following assumptions are made:

- Motion is only in the vertical direction (1D model)

- The rocket is treated as a rigid body

- Gravity is constant (g = 9.81 m/s^2)

- No aerodynamic drag is considered initially

- Mass of the rocket is constant

- Thrust acts only in the vertical direction

Mathematical Model:

Newton’s Second Law

F = m*a

For the rocket:

mdv/dt = T – mg

Where:

T = thrust

mg = gravitational force

Acceleration

a = (T – mg)/m

Velocity

dv/dt = a

Position(altitude)

dh/dt = v

The rocket motion is governed by second-order differential equations, which are solved numerically using a time stepping approach.

Numerical Implementation (MATLAB):

The system is simulated using a discrete-time numerical integration approach with a fixed time step.

Steps:

- Initialize parameters (mass, gravity, step)

- Set initial conditions (velocity = 0, altitude = 0)

- Compute acceleration using force equation

- Update velocity using:

v = v + a * dt

h = h + v * dt

A for loop is used to simulate the system over time.

Open-Loop System (No Control):

Initially, the rocket motion was simulated using constant thrust without feedback control.

Observations:

- If thrust > weight – rocket accelerates upward

- If thrust = weight – rocket hovers

- If thrust < weight – rocket falls

The system cannot reach a specific altitude without feedback control.

Closed Loop Control:

Control Objective

The goal is to make the rocket reach a desired altitude (100m) automatically.

Error Definition

error = h desired – h

PID Controller

T = Kp * error + Ki integral error dt + Kd d(error)/dt

- Proportional (Kp) – reacts to current error

- Integral (Ki) – removes steady-state error

- Derivative (Kd) – reduces oscillations

The controller continuously adjusts thrust based on altitude error.

PID Tuning:

Initial PID gains resulted in overshoot and oscillations.

The gains were tuned experimentally to achieve a stable response.

Final Gains

Kp = 1.2

Ki = 0.02

Kd = 2.5

Observation:

- Smooth ride

- Reduced oscillation

- Stable near target altitude

Advanced control strategy (Phase-Based Control):

To improve the realism and performance of the rocket, the control system was extended beyond a basic PID controller into a multi-phase control strategy.

Instead of using a single control rule throughout the flight, the rocket behavior was divided into three distinct phases:

Phase 1: Ascent

- The objective is to reach the desired altitude (100m).

- The controller uses position-based PID control:

error = h target – h

- The thrust is adjusted to minimize error and achieve a smooth rise.

Phase 2: Hover

- Once the rocket reaches near the target (~ 95m), it transitions into hover.

- The goal is to maintain a constant altitude.

- The same position control strategy is used.

- The velocity naturally reduces to zero due to damping.

Phase 3: Descent

- After a fixed time, the rocket begins descent.

- Instead of controlling position, the system switches to velocity control:

error = v target – v

- A constant downward velocity (e.g., -2.5m/s) is targeted for smooth descent.

Switching Between Control Modes:

The transitions between phases are implemented using conditional statements:

- When altitude >= 95m – switch to hover phase

- When time exceeds a set limit – switch to descent phase

At each phase transition:

- Integral term is set

- Derivative term is reset

- Previous error is reset

This prevents instability caused by accumulated past errors.

Enhancements in PID Controller:

To improve the stability and realism of the system, several modifications were introduced:

1. Integral Windup Prevention

The integral term is limited within a fixed range to prevent excessive accumulation:

Integral = limited between [-100, 100]

This ensures that the controller does not produce excessively large thrust values.

2. Derivative Smoothing

This derivative term is prone to noise and sudden spikes. To overcome this, a filtered derivative its used:

- A raw derivative is first calculated

- It is then limited within a safe range

- Finally, smoothing is applied using a weighted average

This results in a more stable response and reduces oscillations.

3. Velocity Damping

During ascent and hover, a damping term is proportional to velocity is included:

-1.5 * v

This helps:

- Reduce oscillations

- Stabilize the system near the target altitude

During descent, this term is removed because it interferes with velocity control.

Thrust Limitation and Gravity Compensation:

The thrust generated by the controller is adjusted as follows:

- Gravity compensation is added:

T = T + mg

- Thrust is constrained within physical limits:

0<= T <= 30

This ensures that the system behaves realistically, as real rockets cannot produce unlimited thrust.

Results and Analysis:

The simulation results are shown in the plots of the altitude and velocity versus time.

Altitude Behavior

- The rocket smoothly rises from ground level

- It stabilizes near the desired altitude

- After the hover phase, it descends gradually

Figure 1: Altitude of the rocket during the simulated flight.

Velocity Behavior

- High initial velocity during takeoff

- Velocity reduces to zero during hover

- Becomes negative during descent, indicating downward motion

Figure 2: Velocity of the rocket during the simulated flight.

Discussion:

The introduction of the phase-based control significantly improved system performance:

- The rocket reaches the desired altitude without excessive oscillation

- Hovering is stable due to damping

- Descent is controlled and smooth using velocity regulation

The system demonstrated how switching control strategies based on operating conditions leads to better performance than a single fixed controller.

Conclusion:

This project successfully demonstrates the simulation and control of a rocket’s vertical motion using PID control in MATLAB.

Key achievements include:

- Accurate modeling of rocket dynamics

- Effective implementation of PID control

- Stable ascent, hover, and descent behavior

- Improved performance through phase-based control

The system is robust, stable, and behaves in accordance with physical principles.

The project demonstrates practical implementation of control systems concepts such as feedback control, stability, and system dynamics.